DOC-04 FILE FORMATS & PARSER SPECIFICATION SVG · DXF · DWG · G-code · DXF Output · PDF Reports

1.  SVG PARSER — svg_parser.py

SVG files are the primary import format. DropNest must handle all real-world SVG complexity.

| SVG Element | Handling | Notes |
| --- | --- | --- |
| <path d='...' | Full parsing — all commands | M,L,H,V,C,Q,A,Z commands |
| <rect> | Convert to 4-vertex polygon | Handle rx/ry for rounded rect → flatten curves |
| <circle> | Flatten to N-gon (N configurable) | Default N=64 for smooth circles |
| <ellipse> | Flatten arc to polyline | tolerance: 0.05mm default |
| <polygon> | Direct vertex extraction | Most common in CAD SVG exports |
| <polyline> | Extract + auto-close if needed | Warn if not closed |
| <g transform='...' | Apply matrix transforms | scale(), rotate(), translate(), matrix() |
| Nested <g> | Recursively flatten groups | Extract leaves only |
| stroke-width | Detect, optionally use as kerf | User can map stroke-width → kerf |
| viewBox | Scale to mm using dpi | Default: 96dpi (SVG spec) |
| Bezier curves | Flatten with adaptive tolerance | Max error = EPSILON setting |
| Arc commands | Convert A command → polyline | Endpoint parameterization → center |

SVG PARSER CODE STRUCTURE — parsers/svg_parser.py

class SVGParser:

def parse(self, filepath: Path) -> List[Part]:

tree = lxml.etree.parse(filepath)

viewbox = self._get_viewbox(tree) # normalize coordinates

scale = self._compute_mm_scale(viewbox) # px → mm

shapes = []

for element in tree.iter(): # walk all elements

polygon = self._element_to_polygon(element, scale)

if polygon: # skip non-shape elements

shapes.append(polygon)

return [self._make_part(p) for p in shapes]

def _flatten_bezier(self, ctrl_pts, tolerance=0.05):

# Recursive subdivision until all segments < tolerance

# Returns polyline approximation

def _apply_transform(self, points, transform_str):

# Parse transform matrix → apply to all vertices

2.  DXF PARSER — dxf_parser.py

DXF is the standard for CNC software. Must handle R12 through R2018 formats.

| DXF Entity | Handling | Library Call |
| --- | --- | --- |
| LWPOLYLINE | Direct vertex extraction | entity.get_points() |
| POLYLINE | 2D vertices, skip 3D flag | entity.points() |
| LINE | Convert to 2-vertex segment | Often needs grouping |
| CIRCLE | Flatten to polygon | Circle(center, radius) → N-gon |
| ARC | Flatten arc segment | Arc(center, r, start°, end°) |
| ELLIPSE | Flatten to polyline | EllipseEntity → sample points |
| SPLINE | Flatten B-spline to polyline | entity.flattening(tolerance) |
| INSERT (block) | Expand block references | Recursively resolve blocks |
| HATCH | Extract boundary loops | entity.paths → BoundaryPath |
| 3DSOLID | Skip with warning | Not supported — need 2D projection |
| LAYER | Preserve per-entity | User can filter by layer |
| COLOR | Preserve per-entity | Color coding in output DXF |

DXF PARSER KEY CODE — parsers/dxf_parser.py

import ezdxf

class DXFParser:

def parse(self, filepath: Path) -> List[Part]:

doc = ezdxf.readfile(str(filepath))

msp = doc.modelspace()

parts = []

for entity in msp:

polygon = self._entity_to_polygon(entity)

if polygon:

parts.append(self._make_part(polygon))

# Group LINE segments into closed polygons

parts += self._group_lines(msp)

return parts

def _entity_to_polygon(self, entity):

dxftype = entity.dxftype()

if dxftype == 'LWPOLYLINE':

pts = [(p[0], p[1]) for p in entity.get_points('xy')]

return pts if entity.is_closed else None

elif dxftype == 'SPLINE':

pts = list(entity.flattening(0.01)) # tolerance 0.01mm

return [(p[0],p[1]) for p in pts]

# ... handle all other types

3.  G-CODE OUTPUT — gcode_writer.py

G-code must be correct for three machine types. Each machine has a profile with different commands.

| Parameter | CNC Router | Laser Cutter | Plasma Cutter |
| --- | --- | --- | --- |
| Units | G21 (mm) | G21 (mm) | G21 (mm) |
| Tool start | M3 Sxxxx (spindle) | M3 Pxx (laser power) | M3 (arc start) |
| Tool stop | M5 | M5 | M5 |
| Rapid move | G0 X Y Z | G0 X Y | G0 X Y |
| Cut move | G1 X Y Z F | G1 X Y F | G1 X Y F |
| Arc CW | G2 | — | G2 |
| Arc CCW | G3 | — | G3 |
| Plunge | G1 Z-depth F_plunge | — | Pierce delay G4 Pxx |
| Retract | G1 Zsafe | — | G0 Z_safe |
| Coolant on | M8 | — | — |
| Air assist | — | M7 | — |
| End program | M30 | M30 | M30 |

G-CODE WRITER STRUCTURE

class GCodeWriter:

def write(self, placements: List[Placement],

profile: MachineProfile) -> str:

lines = []

lines += self._header(profile) # G21, G90, G17, etc.

ordered = tsp_order(placements) # minimize rapid moves

for placement in ordered:

path = self._get_toolpath(placement, profile)

lines += self._emit_path(path, profile)

lines += self._footer(profile) # M5, M30, etc.

return '\n'.join(lines)

def _emit_path(self, path, profile):

lines = []

# Lead-in

if profile.lead_in_type != 'none':

lines += lead_inout.gen_lead_in(path[0], profile)

# Cut moves

lines.append(f'M3 S{profile.spindle_speed}') # tool on

for pt in path:

lines.append(f'G1 X{pt.x:.4f} Y{pt.y:.4f} F{profile.feed_rate}')

lines.append('M5') # tool off

# Lead-out

if profile.lead_out_type != 'none':

lines += lead_inout.gen_lead_out(path[-1], profile)

return lines

4.  PDF REPORT — pdf_report.py

PDF REPORT STRUCTURE (ReportLab)

Page 1: JOB SUMMARY

├── DropNest logo + job name + date

├── Sheet layout image (rendered from canvas at 300dpi)

├── Key stats: waste%, sheets used, total area, efficiency

└── Machine profile used

Page 2: PART LIST

└── Table: Part Name ¦ Qty ¦ Area mm² ¦ Cost ¦ % of Sheet

Page 3: COST BREAKDOWN

├── Material cost per sheet × sheets used

├── Estimated cut time (minutes)

├── Cost per part (area-proportional)

└── Total job cost

Page 4+: INDIVIDUAL SHEET LAYOUTS

└── One page per sheet — full size layout image

with part labels, grain direction arrows, waste highlighted