DOC-05 MANUFACTURING & CNC SPECIFICATION Toolpath · Kerf · Lead-in/out · Machine Profiles · Material Settings

1.  KERF COMPENSATION

Kerf = the width of material removed by the cutting tool. If ignored, all parts are slightly undersized. DropNest compensates by offsetting every polygon outward by kerf/2 before nesting.

KERF COMPENSATION — offset.py

def apply_kerf(polygon: Polygon, kerf_mm: float) -> Polygon:

# Expand polygon outward by kerf/2

# This reserves the kerf width during nesting

offset_dist = kerf_mm / 2.0

expanded = clipper.InflatePaths([polygon.outer], offset_dist, JoinType.Round)

# Holes: shrink by kerf/2 (they get larger after cutting)

shrunk_holes = [clipper.InflatePaths([h], -offset_dist) for h in polygon.holes]

return Polygon(outer=expanded[0], holes=shrunk_holes)

Typical kerf values by machine:

CNC Router (6mm bit): kerf = 6.0mm

CNC Router (3mm bit): kerf = 3.0mm

Laser CO2 (1.5kW): kerf = 0.2mm

Plasma (thin plate): kerf = 1.5–2.5mm

Waterjet: kerf = 1.0–1.5mm

Knife cutter (fabric): kerf = 0.0mm (no material removed)

2.  LEAD-IN / LEAD-OUT — toolpath/lead_inout.py

Lead-in and lead-out are short paths added before and after each cut contour. They prevent start/end marks on parts and protect entry points.

| Type | Description | Best For |
| --- | --- | --- |
| Straight perpendicular | 90° entry into contour at midpoint | CNC router basic |
| Tangential arc | Smooth curved entry matching contour direction | Plasma, laser — no dwell marks |
| Spiral | Helical entry (3D) — plunges while moving | CNC router on thick material |
| Ramp | Gradual Z descent along contour edge | CNC router — avoids full plunge |
| No lead-in | Direct plunge at contour start | Simple jobs, fast cutting |

3.  TOOLPATH ORDERING — toolpath/ordering.py

Cut order matters: bad ordering wastes time on long rapid moves and can warp thin material (thermal). DropNest uses TSP (Travelling Salesman Problem) to find optimal cut sequence.

TSP ORDERING ALGORITHM

def optimize_cut_order(placements: List[Placement]) -> List[Placement]:

# Step 1: Nearest-neighbor greedy solution (fast baseline)

ordered = nearest_neighbor_tsp(placements)

# Step 2: 2-opt improvement (swap pairs of edges if it shortens route)

improved = two_opt(ordered, max_iterations=1000)

# Step 3: Apply thermal spreading if metal material

if config.material.is_metal and config.heat_distribution:

improved = spread_thermal(improved) # avoid adjacent cuts

return improved

Result: Minimizes total rapid-move distance between parts

Typical improvement: 30–60% reduction in machine travel time

4.  MACHINE PROFILES — ALL SETTINGS

| Setting | Router | Laser | Plasma |
| --- | --- | --- | --- |
| Feed rate (cut) | 1000–5000 mm/min | 2000–20000 mm/min | 2000–8000 mm/min |
| Feed rate (rapid) | 5000–30000 mm/min | Max speed | Max speed |
| Plunge rate | 200–800 mm/min | Instant (no Z) | Instant (no Z) |
| Spindle speed | 8000–24000 RPM | Power 0–100% | Amperage |
| Pierce/plunge delay | 0–2 sec | 0 (instant) | 0.3–1.5 sec |
| Safe Z height | 10–50mm above sheet | Fixed focal Z | 5–15mm above |
| Cut depth | Sheet thickness + 0.5mm | — | — |
| Coolant | M7 mist / M8 flood | — | — |
| Air assist | — | M7 on cut | — |
| Kerf default | 3–6mm (tool diameter) | 0.1–0.3mm | 1.5–3mm |
| Lead-in preferred | Ramp or perpendicular | Tangential arc | Tangential arc |

5.  GRAIN DIRECTION — IMPLEMENTATION

Wood and fabric have a grain direction. Parts must be oriented so grain runs a certain way. DropNest locks rotation when grain direction is set.

GRAIN DIRECTION LOGIC

class Part:

grain_angle: Optional[float] # None = free rotation

# e.g. 0.0 = grain must run horizontal (along X axis)

In SA optimizer:

def move_rotate(part, placement):

if part.grain_angle is not None:

# Only allow rotation that keeps grain aligned

allowed = [part.grain_angle, part.grain_angle + 180]

new_angle = random.choice(allowed)

else:

new_angle = random.choice(part.rotation_angles)

return Placement(..., rotation=new_angle)

Display: Canvas shows grain arrows on each part as directional indicator

UI: Per-part toggle in Part Settings panel → lock grain checkbox

6.  MATERIAL DEFECT ZONES

DEFECT ZONE IMPLEMENTATION

A defect zone is a polygon on the sheet that cannot be used.

Examples: knots in wood, scratches in metal, worn areas in fabric.

Implementation:

1. User draws rectangle(s) on canvas → DefectZone polygons added to Sheet

2. IFP computation: sheet polygon has defect zones as holes

IFP = Clipper.Difference(sheet_rect, union_of_defect_zones)

3. Result: parts are automatically placed around defect zones

4. Defect zones shown as red hatched areas in canvas UI

Sheet defect model:

sheet.defect_zones: List[Polygon] # added via UI

effective_sheet = sheet_polygon with holes at defect_zones

All IFP and placement computations use effective_sheet