DOC-07 DATABASE SCHEMA & DATA MODELS SQLite schema · Pydantic models · Project save/load · Material library

1.  COMPLETE SQLite SCHEMA — database/schema.sql

FULL DATABASE SCHEMA

-- Projects table: one row per saved job

CREATE TABLE projects (

id TEXT PRIMARY KEY, -- UUID

name TEXT NOT NULL,

created_at INTEGER NOT NULL, -- Unix timestamp

updated_at INTEGER NOT NULL,

version INTEGER DEFAULT 1, -- for undo/versioning

config_json TEXT NOT NULL, -- NestConfig as JSON

status TEXT DEFAULT 'draft' -- draft¦complete¦archived

);

-- Parts table: imported shapes per project

CREATE TABLE parts (

id TEXT PRIMARY KEY,

project_id TEXT REFERENCES projects(id) ON DELETE CASCADE,

name TEXT NOT NULL,

quantity INTEGER DEFAULT 1,

polygon_json TEXT NOT NULL, -- Polygon model as JSON

area_mm2 REAL NOT NULL,

grain_angle REAL, -- NULL = free rotation

rotation_step REAL DEFAULT 90.0,

allow_flip INTEGER DEFAULT 0,

metadata_json TEXT -- layer, color, source_file

);

-- Sheets table: material sheets per project

CREATE TABLE sheets (

id TEXT PRIMARY KEY,

project_id TEXT REFERENCES projects(id) ON DELETE CASCADE,

width_mm REAL NOT NULL,

height_mm REAL NOT NULL,

material_id TEXT REFERENCES materials(id),

cost REAL DEFAULT 0.0,

defects_json TEXT -- list of defect zone polygons

);

-- Nest results: saved nesting outputs

CREATE TABLE nest_results (

id TEXT PRIMARY KEY,

project_id TEXT REFERENCES projects(id) ON DELETE CASCADE,

created_at INTEGER NOT NULL,

placements_json TEXT NOT NULL, -- List[Placement] as JSON

waste_pct REAL NOT NULL,

sheets_used INTEGER NOT NULL,

total_cost REAL NOT NULL,

cut_time_sec REAL NOT NULL,

algorithm TEXT, -- 'blf', 'sa', 'hybrid'

iterations INTEGER

);

-- Materials library: reusable material presets

CREATE TABLE materials (

id TEXT PRIMARY KEY,

name TEXT NOT NULL UNIQUE,

category TEXT, -- wood/metal/fabric/plastic

width_mm REAL NOT NULL,

height_mm REAL NOT NULL,

thickness_mm REAL,

cost_per_sheet REAL DEFAULT 0.0,

has_grain INTEGER DEFAULT 0,

grain_angle REAL DEFAULT 0.0,

notes TEXT

);

-- Machine profiles: CNC machine settings

CREATE TABLE machine_profiles (

id TEXT PRIMARY KEY,

name TEXT NOT NULL UNIQUE,

machine_type TEXT NOT NULL, -- router¦laser¦plasma¦waterjet

feed_rate REAL NOT NULL, -- mm/min

rapid_rate REAL,

spindle_speed REAL,

kerf_mm REAL DEFAULT 0.0,

plunge_rate REAL,

pierce_delay REAL DEFAULT 0.0,

safe_z_mm REAL DEFAULT 10.0,

settings_json TEXT -- extra machine-specific settings

);

-- Job history: optimization run history for AI learning

CREATE TABLE job_history (

id TEXT PRIMARY KEY,

project_id TEXT,

part_count INTEGER,

avg_part_area REAL,

algorithm TEXT,

sa_config_json TEXT,

final_waste_pct REAL,

duration_sec REAL,

created_at INTEGER

);

-- Indexes for performance

CREATE INDEX idx_parts_project ON parts(project_id);

CREATE INDEX idx_sheets_project ON sheets(project_id);

CREATE INDEX idx_results_project ON nest_results(project_id);

CREATE INDEX idx_history_date ON job_history(created_at DESC);

2.  PYDANTIC DATA MODELS — backend/models/schemas.py

ALL PYDANTIC MODELS

from pydantic import BaseModel

from typing import List, Optional

class Point(BaseModel):

x: float

y: float

class AABB(BaseModel):

x_min: float; y_min: float; x_max: float; y_max: float

class Polygon(BaseModel):

id: str

outer: List[List[float]] # [[x,y], ...]

holes: List[List[List[float]]] = []

area: float

bbox: AABB

class Part(BaseModel):

id: str

name: str

polygon: Polygon

quantity: int = 1

grain_angle: Optional[float] = None

rotation_step: float = 90.0

allow_flip: bool = False

metadata: dict = {}

class Sheet(BaseModel):

id: str

width: float

height: float

material: str = 'unknown'

cost: float = 0.0

defect_zones: List[Polygon] = []

class Placement(BaseModel):

part_id: str

part_copy_index: int = 0 # for multi-quantity parts

sheet_id: str

x: float

y: float

rotation: float

flipped: bool = False

class NestConfig(BaseModel):

mode: str = 'balanced' # speed¦balanced¦quality

rotation_step: float = 90.0

spacing_mm: float = 2.0 # minimum gap between parts

allow_flip: bool = False

sa_T_max: float = 200.0

sa_T_min: float = 0.1

sa_cooling: float = 0.995

sa_max_iter: int = 20000

parallel_workers: int = 4

class NestResult(BaseModel):

id: str

placements: List[Placement]

sheets_used: int

waste_pct: float

total_cost: float

cut_time_sec: float

algorithm: str

iterations: int

class MachineProfile(BaseModel):

name: str

machine_type: str # router¦laser¦plasma

feed_rate: float

spindle_speed: Optional[float] = None

kerf_mm: float = 0.0

plunge_rate: Optional[float] = None

pierce_delay: float = 0.0

safe_z_mm: float = 10.0

lead_in_type: str = 'tangential'

lead_in_length_mm: float = 5.0

3.  DEFAULT MATERIAL LIBRARY — Pre-loaded on first launch

| Material Name | Width×Height (mm) | Cost | Category | Grain |
| --- | --- | --- | --- | --- |
| MDF 18mm (8×4 sheet) | 2440 × 1220 | $25 | Wood | No |
| MDF 12mm (8×4 sheet) | 2440 × 1220 | $18 | Wood | No |
| Plywood 18mm (8×4) | 2440 × 1220 | $35 | Wood | Yes — 0° |
| Plywood 12mm (8×4) | 2440 × 1220 | $28 | Wood | Yes — 0° |
| Aluminum 3mm (8×4) | 2440 × 1220 | $85 | Metal | No |
| Mild Steel 3mm (8×4) | 2440 × 1220 | $55 | Metal | No |
| Acrylic 6mm (8×4) | 2440 × 1220 | $120 | Plastic | No |
| Fabric (1.5m roll) | 1500 × 3000 | $40 | Fabric | Yes — 0° |
| Leather (hide approx) | 1200 × 900 | $150 | Leather | No |
| Custom (user defined) | User set | User set | Any | Optional |

4.  PROJECT SAVE/LOAD — project_db.py

SAVE AND LOAD IMPLEMENTATION

class ProjectDB:

def __init__(self, db_path: Path):

self.db_path = db_path

async def save_project(self, name: str, parts: List[Part],

sheets: List[Sheet], config: NestConfig,

result: Optional[NestResult] = None) -> str:

project_id = str(uuid4())

async with aiosqlite.connect(self.db_path) as db:

await db.execute(INSERT_PROJECT, (project_id, name, ...))

for part in parts:

await db.execute(INSERT_PART, (part.id, project_id, ...))

for sheet in sheets:

await db.execute(INSERT_SHEET, (sheet.id, project_id, ...))

if result:

await db.execute(INSERT_RESULT, (result.id, project_id, ...))

await db.commit()

return project_id

async def load_project(self, project_id: str) -> dict:

async with aiosqlite.connect(self.db_path) as db:

project = await db.execute_fetchone(SELECT_PROJECT, (project_id,))

parts = await db.execute_fetchall(SELECT_PARTS, (project_id,))

sheets = await db.execute_fetchall(SELECT_SHEETS, (project_id,))

result = await db.execute_fetchone(SELECT_LATEST_RESULT, (project_id,))

return { 'project': project, 'parts': parts,

'sheets': sheets, 'result': result }

async def list_projects(self) -> List[dict]:

# Returns id, name, created_at, part_count, waste_pct of latest result